NAME : GOKUL BARADWAJ C

REG NO. : 192512073

COURSE : DIGITAL SIGNAL PROCESSING

SIMULATION EXPERIMENT – 1

1.Design, Realization and Performance Analysis of Butterworth IIR Low-Pass and High-Pass Digital Filters Using MATLAB

Program code :

clear;

clc;

close all;

% Design Parameters

Fs = 8000;      	% Sampling frequency in Hz

Ap = 1;         	% Passband ripple in dB

As = 40;	        % Stopband attenuation in dB

% 1. Low-Pass Filter Design

fp_lp = 1000;   	% LPF Passband frequency (Hz)

fs_lp = 1500;   	% LPF Stopband frequency (Hz)

wp_lp = fp_lp / (Fs/2);

ws_lp = fs_lp / (Fs/2);

[N_lp, Wn_lp] = buttord(wp_lp, ws_lp, Ap, As);

[b_lp, a_lp] = butter(N_lp, Wn_lp, 'low');

% 2. High-Pass Filter Design

fp_hp = 2500;   	% HPF Passband frequency (Hz)

fs_hp = 1800;   	% HPF Stopband frequency (Hz)

wp_hp = fp_hp / (Fs/2);

ws_hp = fs_hp / (Fs/2);

[N_hp, Wn_hp] = buttord(wp_hp, ws_hp, Ap, As);

[b_hp, a_hp] = butter(N_hp, Wn_hp, 'high');

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf('=== EXPERIMENT 1: BUTTERWORTH IIR FILTERS ===\n\n');

fprintf('--- LOW-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_lp): %d\n', N_lp);

fprintf('3-dB Cutoff Frequency (Wn_lp): %.4f Hz\n', Wn_lp * (Fs/2));

fprintf('Numerator Coefficients (b_lp):\n');

disp(b_lp);

fprintf('Denominator Coefficients (a_lp):\n');

disp(a_lp);

fprintf('--- HIGH-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_hp): %d\n', N_hp);

fprintf('3-dB Cutoff Frequency (Wn_hp): %.4f Hz\n', Wn_hp * (Fs/2));

fprintf('Numerator Coefficients (b_hp):\n');

disp(b_hp);

fprintf('Denominator Coefficients (a_hp):\n');

disp(a_hp);

% Frequency Response Calculation

[H_lp, f] = freqz(b_lp, a_lp, 1024, Fs);

[H_hp, ~] = freqz(b_hp, a_hp, 1024, Fs);

% Plotting

figure('Name', 'Butterworth IIR LPF and HPF');

subplot(2,2,1);

plot(f, 20*log10(abs(H_lp)), 'b', 'LineWidth', 1.5); grid on;

title(['Butterworth LPF Magnitude (Order N = ', num2str(N_lp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]);

subplot(2,2,2);

plot(f, angle(H_lp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Butterworth LPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

subplot(2,2,3);

plot(f, 20*log10(abs(H_hp)), 'b', 'LineWidth', 1.5); grid on;

title(['Butterworth HPF Magnitude (Order N = ', num2str(N_hp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-80 5]);

subplot(2,2,4);

plot(f, angle(H_hp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Butterworth HPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

Output:

STUDENT RELECTION :

This experiment helped me connect theoretical IIR filter concepts with practical implementation in MATLAB. Using buttord and butter showed how frequency normalization and transition bandwidth directly determine the filter order (N). Analyzing the magnitude and phase plots confirmed the Butterworth filter's maximally flat passband, smooth roll-off, and non-linear phase response.

RESULT:

LOW-PASS FILTER ---
 Calculated Filter Order (N_lp): 12
 3-dB Cutoff Frequency (Wn_lp): 1087.8354 Hz
 Numerator Coefficients (b_lp):
 	0.0000	0.0000    0.0002	0.0006	0.0013    0.0021	0.0024	0.0021    0.0013	0.0006	0.0002    0.0000	0.0000

 Denominator Coefficients (a_lp):
 	1.0000   -5.4622   14.6771  -25.1616   30.3497  -26.9537   17.9862   -9.0534    3.4018   -0.9283	0.1743   -0.0202	0.0011

--- HIGH-PASS FILTER ---
 Calculated Filter Order (N_hp): 10
 3-dB Cutoff Frequency (Wn_hp): 2379.7556 Hz
 Numerator Coefficients (b_hp):
 	0.0006   -0.0055    0.0248   -0.0662	0.1159   -0.1390	0.1159   -0.0662    0.0248   -0.0055	0.0006

 Denominator Coefficients (a_hp):
 	1.0000	1.8915    2.8535	2.6014	1.8704    0.9534	0.3726	0.1030    0.0199	0.0023	0.0001