NAME : GOKUL BARADWAJ C

REG NO. : 192512073

COURSE : DIGITAL SIGNAL PROCESSING

SIMULATION EXPERIMENT – 2

2. Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

Program Code:

clear; clc; close all;

% Design Parameters

Fs = 10000;         % Sampling frequency in Hz

Ap = 0.5;       	% Passband ripple in dB

As = 30;        	% Stopband attenuation in dB

% 1. Low-Pass Filter Design

fp_lp = 1500;   	% Passband frequency (Hz)

fs_lp = 2000;   	% Stopband frequency (Hz)

wp_lp = fp_lp / (Fs/2);

ws_lp = fs_lp / (Fs/2);

[N_lp, Wn_lp] = cheb1ord(wp_lp, ws_lp, Ap, As);

[b_lp, a_lp] = cheby1(N_lp, Ap, Wn_lp, 'low');

% 2. High-Pass Filter Design

fp_hp = 2500;   	% Passband frequency (Hz)

fs_hp = 1800;   	% Stopband frequency (Hz)

wp_hp = fp_hp / (Fs/2);

ws_hp = fs_hp / (Fs/2);

[N_hp, Wn_hp] = cheb1ord(wp_hp, ws_hp, Ap, As);

[b_hp, a_hp] = cheby1(N_hp, Ap, Wn_hp, 'high');

% ==========================================

% DISPLAY NUMERICAL OUTPUTS IN COMMAND WINDOW

% ==========================================

fprintf('=== EXPERIMENT 2: CHEBYSHEV TYPE-I IIR FILTERS ===\n\n');

fprintf('--- CHEBYSHEV LOW-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_lp): %d\n', N_lp);

fprintf('Passband Edge Cutoff Frequency (Wn_lp): %.4f Hz\n', Wn_lp * (Fs/2));

fprintf('Numerator Coefficients (b_lp):\n');

disp(b_lp);

fprintf('Denominator Coefficients (a_lp):\n');

disp(a_lp);

fprintf('--- CHEBYSHEV HIGH-PASS FILTER ---\n');

fprintf('Calculated Filter Order (N_hp): %d\n', N_hp);

fprintf('Passband Edge Cutoff Frequency (Wn_hp): %.4f Hz\n', Wn_hp * (Fs/2));

fprintf('Numerator Coefficients (b_hp):\n');

disp(b_hp);

fprintf('Denominator Coefficients (a_hp):\n');

disp(a_hp);

% Frequency Response Calculation

[H_lp, f] = freqz(b_lp, a_lp, 1024, Fs);

[H_hp, ~] = freqz(b_hp, a_hp, 1024, Fs);

% Plotting

figure('Name', 'Chebyshev Type-I LPF and HPF Responses');

subplot(2,2,1);

plot(f, 20*log10(abs(H_lp)), 'b', 'LineWidth', 1.5); grid on;

title(['Chebyshev LPF Magnitude (Order N = ', num2str(N_lp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-60 5]);

subplot(2,2,2);

plot(f, angle(H_lp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Chebyshev LPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

subplot(2,2,3);

plot(f, 20*log10(abs(H_hp)), 'b', 'LineWidth', 1.5); grid on;

title(['Chebyshev HPF Magnitude (Order N = ', num2str(N_hp), ')']);

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-60 5]);

subplot(2,2,4);

plot(f, angle(H_hp)*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Chebyshev HPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

Output:

STUDENT RELECTION :

his experiment provided practical insight into designing Chebyshev Type-I IIR low-pass and high-pass filters in MATLAB using cheb1ord and cheby1. Analyzing the frequency response demonstrated the core trade-off of Chebyshev Type-I filters: accepting equiripple variations in the passband achieves a much steeper transition roll-off and requires a lower filter order (N) compared to a Butterworth filter with identical specifications.

RESULT :

--- CHEBYSHEV LOW-PASS FILTER ---

Calculated Filter Order (N_lp): 6

Passband Edge Cutoff Frequency (Wn_lp): 1500.0000 Hz

Numerator Coefficients (b_lp):

0.0006    0.0038    0.0095    0.0127    0.0095    0.0038    0.0006

Denominator Coefficients (a_lp):

1.0000   -3.7650    6.8198   -7.3274    4.8717   -1.8927    0.3368

--- CHEBYSHEV HIGH-PASS FILTER ---

Calculated Filter Order (N_hp): 6

Passband Edge Cutoff Frequency (Wn_hp): 2500.0000 Hz

Numerator Coefficients (b_hp):

0.0117   -0.0704    0.1761   -0.2348    0.1761   -0.0704    0.0117

Denominator Coefficients (a_hp):

1.0000    1.3568    2.1347    1.8511    1.2978    0.5937    0.1650